for your goal on Scaleway, MuJoCo + MJX is the faster path. Isaac Lab's setup overhead is real and the USD format conversion is extra friction. MuJoCo is literally pip install mujoco, loads your URDF directly, and the AMP paper that pioneered video imitation was built on MuJoCo. The brachiation and acrobatics research all uses MuJoCo.

Files you need beyond the URDF

ape_robot/

├── urdf/

│   └── ape.urdf.xacro        ← main robot description

├── meshes/

│   └── roarm_m3/             ← STL files (already in your repo)

│       ├── base_link.stl

│       ├── link1.stl

│       └── ...

└── mjcf/

└── ape.xml               ← MuJoCo version (auto-converted from URDF)

The STL mesh files are already in your repo under src/roarm_main/roarm_description/meshes/roarm_m3/. You reuse them for all 4 arms.

URDF basics — what you actually need to know

A URDF is just two things repeating:

<!-- LINK = a rigid body -->

<link name="torso">

<inertial>

<mass value="1.5"/>                    <!-- kg -->

<inertia ixx="0.05" iyy="0.05" izz="0.05" ixy="0" ixz="0" iyz="0"/>

</inertial>

<visual>

<geometry><box size="0.3 0.2 0.1"/></geometry>   <!-- or mesh -->

</visual>

<collision>

<geometry><box size="0.3 0.2 0.1"/></geometry>

</collision>

</link>

<!-- JOINT = connects two links -->

<joint name="fl_arm_mount" type="fixed">

<parent link="torso"/>

<child link="fl_base_link"/>

<origin xyz="0.12 0.10 -0.05" rpy="0 0.5 0"/>

<!--         ↑ position        ↑ rotation  -->

</joint>

That's it. Everything else is repetition of those two patterns.

How to orient arms in the right direction

The rpy in the mount joint is your steering wheel. In radians:

rpy="0 0 0"       → arm points straight up

rpy="0 1.5708 0"  → arm points forward (rotated 90° around Y)

rpy="0 0.7854 0"  → arm angled 45° forward-down  ← good for front legs

rpy="0 -0.7854 0" → arm angled 45° backward-down ← good for back legs

For a quadruped ape stance, you want arms pointing outward and downward so the gripper tips touch the ground. Visualize it, tweak the angle, re-visualize — it's fast iteration.

Tools to build and verify

Fastest feedback loop — no ROS needed:

# verify.py — just run this

import mujoco

import mujoco.viewer

model = mujoco.MjModel.from_xml_path("ape.urdf")  # loads URDF directly

data = mujoco.MjData(model)

with mujoco.viewer.launch_passive(model, data) as viewer:

while viewer.is_running():

mujoco.mj_step(model, data)

viewer.sync()

If it loads → structure is valid. If it falls apart → physics/inertia problem. You can drag joints around in the MuJoCo viewer interactively.

Other tools:

- Foxglove Studio — drag-drop URDF visualizer, no ROS, runs in browser

- check_urdf your_file.urdf — ROS command line validator, catches XML errors

- VSCode with the ROS extension — syntax highlighting and inline error checking for xacro

The build order I'd follow:

- Convert roarm_m3.xacro to a prefixed macro — I do this, takes 20 min

- Write ape.urdf.xacro — torso + 4 arm instances

- Load in MuJoCo viewer, check it looks right, adjust mount angles

- Add contact points at gripper tips (so MuJoCo knows those are feet)

- Test physics — drop it, see if it collapses realistically

- Then start RL

# Task: Build Robotic Ape URDF for MuJoCo

Goal: Create a robot description file for a 4-limbed robotic ape — one torso with 4x RoArm M3 arms as limbs — that loads and simulates correctly in MuJoCo.

Output files (create these in /home/tony/roarm_ws/ape_sim/)

ape_sim/

├── roarm_m3_macro.xacro     ← single arm as reusable macro

├── ape_robot.urdf.xacro     ← full robot: torso + 4 arms

└── verify_ape.py            ← loads in MuJoCo, opens viewer



What MuJoCo requires from a URDF

- Pure XML — no xacro syntax. MuJoCo cannot read .xacro directly. The xacro must be compiled to plain URDF first:

pip install xacro

xacro ape_robot.urdf.xacro > ape_robot.urdf

- 
The verify script should do this automatically.

- No $(find package_name) in mesh paths. That's a ROS-only feature. Use absolute paths instead:

<!-- wrong -->

<mesh filename="file://$(find roarm_description)/meshes/roarm_m3/base_link.stl"/>

<!-- right — define once as a xacro property -->

<xacro:property name="mesh_base"

value="/home/tony/roarm_ws/src/roarm_main/roarm_description/meshes/roarm_m3"/>

<mesh filename="file://${mesh_base}/base_link.stl" scale="0.001 0.001 0.001"/>

- 
All link and joint names must be unique across the whole robot. Four arms means four copies of base_link, link1, link2, etc. — they will all clash. Fix with a prefix parameter in the macro:

<xacro:macro name="roarm_m3" params="prefix parent mesh_base *mount_origin">

<joint name="${prefix}mount" type="fixed">

<parent link="${parent}"/>

<child link="${prefix}base_link"/>

<xacro:insert_block name="mount_origin"/>

</joint>

<link name="${prefix}base_link"> ... </link>

<link name="${prefix}link1"> ... </link>

<!-- every link and joint name gets ${prefix} prepended -->

</xacro:macro>

- 
Called like:

<xacro:roarm_m3 prefix="fl_" parent="torso" mesh_base="${mesh_base}">

<origin xyz="0.10 0.09 -0.04" rpy="3.14159 0.15 0.15"/>

</xacro:roarm_m3>

- 
No world link, no world_to_base_link joint. In the original arm xacro there's a fixed world link at the root. Remove it. The torso becomes the root link. MuJoCo automatically treats an unconnected root link as a free-floating body (6 DOF in space).

- Inertia values must be non-zero. The existing arm xacro has real inertia values — copy them exactly. A zero inertia will crash MuJoCo.

- Effort and velocity must be non-zero. The original xacro has effort="0" velocity="0" on every joint. MuJoCo needs real values. Use these placeholders until real motor specs arrive:

effort="10" velocity="3.14"



Source files to work from

All inertia, mass, mesh, and joint data already exists — don't reinvent it:

| File | Use it for |
| --- | --- |
| roarm_m3.xacro | All link/joint definitions — copy and prefix them |
| src/roarm_main/roarm_description/meshes/roarm_m3/ | STL mesh files — 8 files, reuse for all 4 arms |

Torso link

Define a simple box. No mesh needed — a box primitive is fine for now:

<link name="torso">

<inertial>

<origin xyz="0 0 0" rpy="0 0 0"/>

<mass value="1.5"/>

<inertia ixx="0.0058" ixy="0" ixz="0"

iyy="0.0121" iyz="0" izz="0.0163"/>

</inertial>

<visual>

<geometry><box size="0.30 0.20 0.08"/></geometry>

</visual>

<collision>

<geometry><box size="0.30 0.20 0.08"/></geometry>

</collision>

</link>



Arm mount positions and orientations

Arms attach to the torso corners pointing downward (flipped 180° from normal upright stance). rpy="3.14159 ..." flips the arm so it hangs down. The slight pitch/roll angles give a natural outward stance — these are starting points, tune in the viewer.

| Limb | xyz on torso | rpy |
| --- | --- | --- |
| front-left (fl_) | 0.10 0.09 -0.04 | 3.14159 0.15 0.15 |
| front-right (fr_) | 0.10 -0.09 -0.04 | 3.14159 0.15 -0.15 |
| back-left (bl_) | -0.10 0.09 -0.04 | 3.14159 -0.15 0.15 |
| back-right (br_) | -0.10 -0.09 -0.04 | 3.14159 -0.15 -0.15 |

Verify script requirements

verify_ape.py should:

- Run xacro ape_robot.urdf.xacro via subprocess to get the compiled URDF

- Load it with mujoco.MjModel.from_xml_string(...)

- Open the interactive viewer with mujoco.viewer.launch_passive(...)

- Step the simulation in a loop so gravity runs and you can watch it fall/settle

Install requirements:

pip install mujoco numpy

pip install xacro



Definition of done

- xacro ape_robot.urdf.xacro runs without errors

- python verify_ape.py opens the MuJoCo viewer showing the robot

- Robot has a torso, 4 visible arms hanging downward from corners

- Dropping it (gravity on) doesn't immediately explode or freeze — it falls and settles

- No duplicate link/joint name errors in MuJoCo output

One URDF file that describes the robotic ape (torso + 4 RoArm M3 arms as limbs) and loads correctly in MuJoCo. That's it.

She figures out the how. You verify by running her script and seeing the robot appear in the MuJoCo viewer and fall under gravity without exploding.

MuJoCo loads URDF (plain XML). It doesn't understand xacro.

Xacro is just a preprocessor — it lets you write reusable templates with variables and macros, then compiles them to a plain URDF. You use xacro to avoid copy-pasting the same arm 4 times.

ape.urdf.xacro  →  [xacro compiles it]  →  ape.urdf  →  MuJoCo loads this

So the workflow is: write xacro (easier to maintain), compile to URDF, feed URDF to MuJoCo.

If she's comfortable writing raw XML without macros, she can skip xacro and write a pure URDF directly — it'll just be longer and more repetitive. Either way, what MuJoCo sees is URDF.